fMRI Basics

# Directory Structure

## Data

Now that you’ve logged onto the system, its worth having a quick look around to familiarise yourself with the directory structure.

Your network home space will be /home/<username>

Your imaging space will be /imaging/<username>

The /imaging tree also contains lots of other useful stuff such as the SPM installation (/imaging/local/software).

When raw data is transferred from the scanner, it ends up in /mridata/cbu. Within this directory, each subject has their own directory (don’t try to ls /mridata/cbu – it could take a long time!)

The format for each subject’s directory name is

/mridata/cbu/<subject-ID>_<project-ID>

e.g.

/mridata/cbu/CBU090250_MR09002

Within each subject’s directory, there’s usually a single session directory, the name of which is derived from the time the data set was collected, e.g.

e.g.

/mridata/cbu/CBU080705_CBU080705/20080826_152343

Within each session directory, there will be a number of directories containing the data from each of the scans you collected. These will be in the format <scan-ID>_<scan-name>, e.g. inside /mridata/cbu/CBU080705_CBU080705/ 20080826_152343/ you can see several folders including:

./Series_001_CBU_Localiser

./Series_002_CBU_MPRAGE

./Series_003_CS_MEP5_16SL

./Series_006_CBU_FieldMapping

Some of the most frequently used scan types are:

CBU_Localiser = a quick pilot scan taken at the beginning of each session. Collects 1 axial, 1 saggital, and 1 coronal slice, and is used to plan the acquisition matrix for the structural scan.

CBU_MPRAGE – the high resolution T1 weighted structural scan, usually run second and used to plan the functional scans.

CBU_FieldMapping – a map of the magnetic field strength within the scanner

CBU_EPI – the default functional image scan, a T2* weighted gradient echo sequence.

## aa

AA provides a powerful batching system with lots of useful features, for example it is restartable (i.e. remembers where it was up to if it gets interrupted), provides multi-level, interlinked diagnostics, supports 4D NIFTI and can handle parallel processing of multiple subjects/sessions.

For more information, have a look at:

http://imaging.mrc-cbu.cam.ac.uk/imaging/AA

aa anatomy

# Starting SPM/aa

Launch MATLAB 2012b

matlab_r2012b

Setup:

addpath /hpc-software/matlab/cbu

addpath /imaging/local/software/spm_cbu_svn/releases/spm12_latest

addpath /imaging/local/software/AA/realease-5.0.0

aa_ver5

SPM and aa are essentially large collections of Matlab scripts, and you can integrate these into your own custom analyses as needed.

# Pre Processing

## 0. Prepare User Master Script

All the pre-processing steps described below are included in a batch script you can find in your imaging space at:

/imaging/train0Xlinux/Workshop/Material/4_aa

If you cannot find them there, you can always grab them from:

/imaging/ta02/Workshop/Metarial_all/4_aa

If you’ve not already done so, make a directory in your imaging space for workshop related stuff. In either matlab or a terminal window type:

mkdir /imaging/<username>/Workshop/Material/4_aa

addpath /imaging/<username>/Workshop/Material/4_aa

Then copy the script over to your local directory. In a terminal window, type:

UMS: cp /imaging/ta02/Workshop/Metarial_all/4_aa/aa_user_fmri.m /imaging/<username>/Workshop/Material/4_aa

Tasklist: cp /imaging/ta02/Workshop/Metarial_all/4_aa /aap_tasklist_fmri.xml /imaging/<username>/Workshop/Material/4_aa

ROIs: cp /imaging/ta02/Workshop/Metarial_all/4_aa /S*.nii /imaging/<username>/Workshop/Material/4_aa

Once you’ve done this, open the script from the matlab command window by typing:

edit  aa_user_fmri.m

The data set we’re going to be working on comes from a localiser task where people moved their right finger. The ID of the subject we’re going to be analysing is 90973 and 90979, and the raw data are located in:

/mridata/cbu/CBU090973_MR09032/20090907_105552

/mridata/cbu/CBU090979_MR09032/20090908_083331

The script is set up to create sub directories within aap.acq_details.root/aap.directory_conventions.analysisid for each module and subject

As the pre-processing progresses, you’ll notice that various prefixes are added to the images. When the functional data is first imported, the images will have the prefix ‘f’. Realignment (+unwarping) will change it to ‘uf’. If you do Slice timing adds ‘a’, normalisation ‘w’, and smoothing ‘s’. At the end of the pre-processing pipeline, you end up with a series of images with prefixes like ‘swauf’ (=smoothed, normalised, slice time corrected, unwarped), ‘wauf’, ‘sauf’ etc depending on exactly which pre-processing steps you carry out.

But you do not have to worry about them because the aa stream-concept will deal with them.

As you analyse more and more data, the temptation becomes just to run the preprocessing automatically and not really check the results. Whilst the typical preprocessing pipeline is fairly robust, it’s always worth spending some time checking your data (report, diagnostics in the module directory). At the very least it’s worth having a look at the mean and variance images for each session, the results from tsdiffana, the plot generated for the raw data right after conversion and after RU and Slice timing, normalisation, and smoothing.

You can get (not always, though ) extra information about each of the spm routines/aa modules by typing help <function name> in the Matlab Command Window.

To get a better idea of what’s going on at each line, you can run the script one line at a time. Put a breakpoint on the first line (click the “-“ sign on the left of the Matlab editor window next to line 18. A red dot should appear). Press F5 to run the script, then press F10 to run each line in turn. To see the contents of variables at each step, hover the mouse cursor over the variable name. As you step through the script, read the comments (in green) to get a better idea of what’s happening on each line.

You can also run it section-by-section (%%).

The first few lines of the script define several variables that will be used, for example the paths of various directories etc. Before running anything, change

- line 20 to your e-mail address: aap.options.email = '<your e-mail address>’;

- line 32 to top analysis folder: aap.acq_details.root = '/imaging/<username>/Workshop';

- (optional) line 33 to analysis subfolder: aap.directory_conventions.analysisid = 'aa_fMRI';

- lines 41-46 to add ROIs to the analysis:

aap = aas_addinitialstream(aap,'rois',{...

'/imaging//<username>/Workshop/Material/4_aa/SMC_L.nii',...

'/imaging//<username>/Workshop/Material/4_aa/SMC_R.nii',...

'/imaging//<username>/Workshop/Material/4_aa/SSC_L.nii',...

'/imaging//<username>/Workshop/Material/4_aa/SSC_R.nii'...

});

Let’s run the script and then I show you around.

Navigate analysis folder.

Navigate aaworker folder (qsub). Empty frequently!

## 0.1. Initialisation

Tasklist:

- aamod_checkparameters

- aamod_evaluatesubjectnames

- aamod_study_init

- aamod_newsubj_init

## 1. Copying the data from the central store and converting to nifti:

The data needs to be copied to the local directory and converted from the dicom format into the format used by SPM.

Tasklist:

- aamod_autoidentifyseries_timtrio

- aamod_get_dicom_structural

- aamod_get_dicom_epi

- aamod_get_dicom_fieldmap

- aamod_convert_structural

- aamod_convert_epis

- aamod_convert_fieldmaps

Gui: “Dicom Import”

To get an idea of what each type of scan looks like

- you can click “Display” in the SPM gui, and then select the image you want to display. To look at multiple images at the same time, click “Check Reg”.

- fslview

If you display an image, try getting a feel for the SPM/fslview display interface – click around on the orthogonal plots at the top, try comparing the voxel space and world space views etc.

## 2. Coregister and normalise the structural using DARTEL

The new method, which we're going to use here, is more accurate, and involves segmenting the image into white and grey matter first, then normalising these to tissue specific templates via a group template.

The MPRAGE sequence used for structurals at the CBU often has a noticeable attenuation bias, however, with greater signal at the back of the brain than at the front. This can produce unpredictable results if the bias isn’t corrected first. The solution to this is a 2 pass procedure – first correct the bias, then segment the bias corrected structural.

Tasklist:

aamod_biascorrect_structural

aamod_coreg_extended_1

aamod_segment8_multichan

aamod_dartel_createtemplate

aamod_dartel_norm_write

Gui (not perfect match):

- “Segment” First pass – bias correction only

  - “save bias corrected” only

  - “do not clean up”

  - “custom”: “no affine regularisation”

The procedure will output: bias corrected (“modulated”) structural (prefix ‘m’)

- “Coregister”/“Coreg: Estimate”

  - Reference Image: /imaging/local/software/spm_cbu_svn/releases/spm12_latest/toolbox/OldNorm/T1.nii

  - Source Image: structural

The output of coregistration includes:

  - A new page in the print file showing the results of the coregistration

  - A modification to the header of the structural image to incorporate the applied transformation.

Using Check Reg, bring up the T1 template (/imaging/local/software/spm_cbu_svn/releases/spm12_latest/toolbox/OldNorm/T1.nii) and the structural image (./structural/s*.nii). Click around the images and look at the results of the coregistration.

- “Segment” Second pass  - Normalise using bias corrected structural

  - “Native Tissue”: “Native + Dartel Imported” “grey matter” and “white matter”

  - “Warped Tissue”: “Modulated + Unmodulated” “grey matter” and “white matter”

  - “custom”: For “affine regularisation” select “ICBM template – European Brains”

The procedure will output:

  - A bias corrected version of your structural image (prefix ‘mm’)

  - Segmented grey and white matter images derived from the structural (grey matter = ‘c1m*.nii’, white matter = ‘c2m*.nii’)

  - Normalised segmented grey and white matter images (grey matter = ‘wc1m*.nii’, white matter = ‘wc2m*.nii’)

  - Normalised version of the complete structural image (‘wm*.nii’) – aa only (you can create it with the GUI with following step 7).

  - A mat file containing the parameters to transform the image from its native space to the template space (‘*seg_sn.mat’)

  - A mat file containing the parameters for the inverse transformation (i.e. from template space to individual subject space; ‘*seg_inv_sn.mat’)

Compare the normalised structural and the normalised grey / white matter partitions in aamod_segment8_multichan_00001 using Check Reg. Finally, compare the un-normalised structural and un-normalised grey / white matter images also in aamod_segment8_multichan_00001.

- DARTEL

## 3. Image Diagnostics with tsdiffana

Tasklist: aamod_tsdiffana

Gui: Type tsdiffana at the matlab prompt, and follow the prompts to select the image files.

Once this is finished, you’ll have an image called vmeanf*.nii in Loc. Load these up and have a look at them. As the name suggests, vmeanf shows the variance across all images, with brighter voxels showing higher variance. The variance image should look like a white outline around the edge of the brain and around the ventricles, as this is where you tend to get the highest variance (small head movements can change the tissue type drastically, e.g. changing the contents of a voxel from grey matter to air or csf). If the variance image just looks black, try going to the “colors” menu in the graphics window and choosing effects, brighten.

As well as the mean and variance images, tsdiffana should have printed a set of graphs diagnostic_aamod_tsdiffana.jpg to the first page of the printfile.  For more details about what the graphs show, see http://imaging.mrc-cbu.cam.ac.uk/imaging/DataDiagnostics. The values plotted in these graphs are also saved to a mat file called tsdiff.mat saved in Loc.

## 4. Realignment & Undistort

Tasklist:

- aamod_fieldmap2VDM

- aamod_realignunwarp

Gui:

- Fieldmap toolbox

- “Realign”/”Realign & Unwarp”.

The realignment process will output:

- A new page in the print file showing the translations and rotations applied to each volume (the realignment parameters).

- A file called “rp*.txt”. This contains the realignment parameters.

- An image called “meanf*.nii” in Sess1. This is the mean realigned image.

- A new set of ‘uf*.nii’ images in each session directory. These are the resliced realigned and unwarped images.

- It will also modify the header files of the ‘f*.nii’ images to incorporate the new transformations applied to those images.

Have a look at the graphs of the realignment parameters – especially look for big jumps in parameters. How does the position of any big changes compare to spikes in the tsdiffana graphs? Whereabouts in the time series are the biggest shifts? What’s the overall size of the movements?

## 5. Slice Timing

Tasklist:  aamod_slicetiming

Gui: “Slice Timing”

- For this data, nslices = 32, TR =2, TA =2-(2/32), slice order = [32:-1:1], and reference slice = 2.

The output of slice timing will be a set of images with the prefix ‘auf’

## 6. Coregister EPI to structural via mean EPI

Tasklist:  aamod_coreg_extended_2epi

Gui (not perfect match): “Coregister”/“Coreg: Estimate”

  - Reference Image: structural after coregistered to the template (see Step 2)

  - Source Image: mean EPI

  - Other images: EPI

The output of coregistration includes:

- A new page in the print file showing the results of the coregistration

- A modification to the header of the structural image to incorporate the applied transformation.

Using Check Reg, bring up the mean undistorted EPI image (./<first session>/meanuf*.nii) images and the structural image (./structural/s*.nii). Click around the images and look at the results of the coregistration.

## 7. Apply DARTEL normalisation parameters to the EPI data

Tasklist:  aamod_norm_write_dartel and aamod_norm_write_meanepi_dartel

Gui: “Batch”/“SPM”/”Tools”/”Dartel Tools”/”Normalise to MNI Space”

Output will be normalised versions of the files you’ve selected. These will have the prefix ‘sw’ added to the beginning of the file name.

## 8. Smooth the EPI data

Tasklist:  aamod_smooth

Gui: “Smooth”

Output will be smoothed versions of the files you’ve selected. These will have the prefix ‘s’ added to the beginning of the file name.

Load up ./<session>/swauf*.nii and sswauf*.nii and compare them using check reg.

# Statistics – GLM

## 1. First-level model

Tasklist:  aamod_fisrtlevel_model

GUI: “Specify 1st-level”, “Estimate”